Question:

Two sinusoidal signals of 10 kHz are made input to two channels of an ADC of STM32F103C8T6. Write a code to read the two analogue signals by the ADC in DMA mode and to write the two values at a sampling frequency of 200 kHz. The sampling time is controlled by a timer and samples for a total time of 10 mS have to be collected. Data have to be stored in two arrays

Answer:

#include "stm32f10x.h"

// Define ADC and DMA parameters

#define ADC_CHANNEL_1 ADC_Channel_1

#define ADC_CHANNEL_2 ADC_Channel_2

#define ADC_SAMPLE_RATE 200000 // 200 kHz

#define SAMPLE_TIME_MS 10

#define NUM_SAMPLES (ADC_SAMPLE_RATE * SAMPLE_TIME_MS / 1000)

// Global variables to store ADC data

uint16_t adc_values_channel1[NUM_SAMPLES];

uint16_t adc_values_channel2[NUM_SAMPLES];

uint16_t adc_buffer[2 * NUM_SAMPLES]; // Interleaved buffer for DMA

void configure_clocks(void) {

// Enable peripheral clocks

RCC_APB2PeriphClockCmd(RCC_APB2Periph_GPIOA | RCC_APB2Periph_ADC1 | RCC_APB2Periph_TIM2, ENABLE);

RCC_AHBPeriphClockCmd(RCC_AHBPeriph_DMA1, ENABLE);

}

void configure_gpio(void) {

GPIO_InitTypeDef GPIO_InitStructure;

// Configure PA1 and PA2 as analog inputs

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_1 | GPIO_Pin_2;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AIN;

GPIO_Init(GPIOA, &GPIO_InitStructure);

}

void configure_adc(void) {

ADC_InitTypeDef ADC_InitStructure;

// ADC1 configuration

ADC_InitStructure.ADC_Mode = ADC_Mode_Independent;

ADC_InitStructure.ADC_ScanConvMode = ENABLE; // Enable scan mode for multiple channels

ADC_InitStructure.ADC_ContinuousConvMode = DISABLE; // Disable continuous conversion

ADC_InitStructure.ADC_ExternalTrigConv = ADC_ExternalTrigConv_T2_TRGO; // Timer 2 TRGO as trigger

ADC_InitStructure.ADC_DataAlign = ADC_DataAlign_Right;

ADC_InitStructure.ADC_NbrOfChannel = 2; // Two channels

ADC_Init(ADC1, &ADC_InitStructure);

// Configure ADC channels

ADC_RegularChannelConfig(ADC1, ADC_CHANNEL_1, 1, ADC_SampleTime_1Cycles5);

ADC_RegularChannelConfig(ADC1, ADC_CHANNEL_2, 2, ADC_SampleTime_1Cycles5);

// Enable DMA for ADC

ADC_DMACmd(ADC1, ENABLE);

// Enable ADC1

ADC_Cmd(ADC1, ENABLE);

// Enable ADC1 reset calibration register

ADC_ResetCalibration(ADC1);

// Check the end of ADC1 reset calibration register

while (ADC_GetResetCalibrationStatus(ADC1));

// Start ADC1 calibration

ADC_StartCalibration(ADC1);

// Check the end of ADC1 calibration

while (ADC_GetCalibrationStatus(ADC1));

}

void configure_dma(void) {

DMA_InitTypeDef DMA_InitStructure;

// DMA1 channel1 configuration

DMA_InitStructure.DMA_PeripheralBaseAddr = (uint32_t)&ADC1->DR;

DMA_InitStructure.DMA_MemoryBaseAddr = (uint32_t)adc_buffer;

DMA_InitStructure.DMA_DIR = DMA_DIR_PeripheralSRC;

DMA_InitStructure.DMA_BufferSize = 2 * NUM_SAMPLES;

DMA_InitStructure.DMA_PeripheralInc = DMA_PeripheralInc_Disable;

DMA_InitStructure.DMA_MemoryInc = DMA_MemoryInc_Enable;

DMA_InitStructure.DMA_PeripheralDataSize = DMA_PeripheralDataSize_HalfWord;

DMA_InitStructure.DMA_MemoryDataSize = DMA_MemoryDataSize_HalfWord;

DMA_InitStructure.DMA_Mode = DMA_Mode_Circular; // Circular mode for continuous transfer

DMA_InitStructure.DMA_Priority = DMA_Priority_High;

DMA_InitStructure.DMA_M2M = DMA_M2M_Disable;

DMA_Init(DMA1_Channel1, &DMA_InitStructure);

// Enable DMA1 channel1

DMA_Cmd(DMA1_Channel1, ENABLE);

}

void configure_timer(void) {

TIM_TimeBaseInitTypeDef TIM_TimeBaseStructure;

// Timer 2 configuration for ADC trigger

TIM_TimeBaseStructure.TIM_Period = (SystemCoreClock / ADC_SAMPLE_RATE) - 1; // Calculate period for desired sample rate

TIM_TimeBaseStructure.TIM_Prescaler = 0;

TIM_TimeBaseStructure.TIM_ClockDivision = 0;

TIM_TimeBaseStructure.TIM_CounterMode = TIM_CounterMode_Up;

TIM_TimeBaseInit(TIM2, &TIM_TimeBaseStructure);

// Configure Timer 2 TRGO as ADC trigger

TIM_SelectOutputTrigger(TIM2, TIM_TRGO_Update);

// Enable Timer 2

TIM_Cmd(TIM2, ENABLE);

}

void process_adc_data(void) {

// De-interleave ADC data

for (int i = 0; i < NUM_SAMPLES; i++) {

adc_values_channel1[i] = adc_buffer[2 * i];

adc_values_channel2[i] = adc_buffer[2 * i + 1];

}

//Now, adc_values_channel1 and adc_values_channel2 contain the data.

//Further processing of the data can be done here.

}

int main(void) {

configure_clocks();

configure_gpio();

configure_adc();

configure_dma();

configure_timer();

// Start ADC conversion by enabling the timer.

TIM_Cmd(TIM2, ENABLE);

//Delay for the sampling time.

for(volatile uint32_t i=0; i< (SystemCoreClock/1000 * SAMPLE_TIME_MS); i++){}

//Stop the timer

TIM_Cmd(TIM2, DISABLE);

//Process the ADC data.

process_adc_data();

// Infinite loop

while (1) {

// Your application code here

}

}